Annotated Code Document

CIVE 202

Project 5 – Automation of Steel Beam Calculations Stel Design

Group 10: Tyler Potthoff, Trey Williamson, Cole Greer, Nate Jones

The Code with Scenarios

| import pandas as pd import numpy as np import plotly.express as px import seaborn as sns | Imports Pandas, Numpy, Plotly, Seaborn. |
| --- | --- |
| sns.set_style('whitegrid') sns.set_palette('deep') | Sets the visual style as whitegrid and the color palette as deep. |
| def Ten_Yield_Strength(Ag, Fy, Phi_T, Omega_T): Nom_Load_Pn = Fy * Ag LRFD = Nom_Load_Pn * Phi_T ASD = Nom_Load_Pn / Omega_T return LRFD, ASD | Defines a function that calculates yield strength. Starts by calculating the nominal tensile strength, then calculates the design strength, and then finally the allowable strength. It then sends back the results of LRFD and ASD. |
| scenarios = [ { "Name": "SCENARIO 1: The Solid Pass", "Pd": 40, "Pl": 120, "Fy": 50, "Fu": 65, "Size": "W10x30", "Ag": 8.84, "bf": 5.81, "tf": 0.510, "d": 10.5, "ry": 1.37, "x_bar": 0.90, "l": 10, "Phi_T": 0.9, "Omega_T": 1.67 }, { "Name": "SCENARIO 2: Strength Failure (Overloaded)", "Pd": 80, "Pl": 180, "Fy": 50, "Fu": 65, "Size": "W6x15", "Ag": 4.43, "bf": 5.99, "tf": 0.260, "d": 5.99, "ry": 1.46, "x_bar": 0.75, "l": 8, "Phi_T": 0.9, "Omega_T": 1.67 }, { "Name": "SCENARIO 3: Slenderness Failure", "Pd": 10, "Pl": 20, "Fy": 50, "Fu": 65, "Size": "W6x9", "Ag": 2.68, "bf": 3.94, "tf": 0.215, "d": 5.90, "ry": 0.905, "x_bar": 0.60, "l": 6, "Phi_T": 0.9, "Omega_T": 1.67 } ] | Creates 3 different scenarios that can be used to test our code. Each variable is assigned a value. |
| db = 0.75 dh = db + 1/16 | Sets bolt diameter as .75 and hole diameter as bolt diameter + 1/16 |
| plot_data = [] | Creates an empty list to store the results of each test for the graph |
| for s in scenarios: # Extract variables for the current scenario Pd = s["Pd"] Pl = s["Pl"] Fy = s["Fy"] Fu = s["Fu"] Size = s["Size"] Ag = s["Ag"] bf = s["bf"] tf = s["tf"] d = s["d"] ry = s["ry"] x_bar = s["x_bar"] l = s["l"] Phi_T = s["Phi_T"] Omega_T = s["Omega_T"] | Repeats command for each scenario given. Sets each variable the value from the scenarios earlier. |
| Pu = 1.2 * Pd + 1.6 * Pl Pa = Pd + Pl LRFD_1, ASD_1 = Ten_Yield_Strength(Ag, Fy, Phi_T, Omega_T) | Calculates the LRFD load, Pu, and the ASD load, Pa. Recalls the function earlier to get the yield capacaties. |
| print ("\n\n========================================================") print (f"RUNNING: {s['Name']}") print ("========================================================") print ("KIEWIT --- W - Shaped Tension Member Testing Report") print ("------------------------------------") print("--------Load Requirements--------") print(f"{'Dead Load (kips):':<30}{Pd:.3f}") print(f"{'Live Load (kips):':<30}{Pl:.3f}") print ("--------Material Properties--------") print ("ASTM A992") print(f"{'Yield Stress (ksi):':<30}{Fy:.3f}") print(f"{'Ultimate Tensile Stress (ksi):':<30}{Fu:.3f}") print ("---------Geometric Properties--------") print(f"{'W Shape Size:':<30}{Size}") print(f"{'Gross Area (in.^2):':<30}{Ag:.3f}") print(f"{'Flange Width (in.):':<30}{bf:.3f}") print(f"{'Flange Thickness (in.):':<30}{tf:.3f}") print(f"{'Depth (in.):':<30}{d:.3f}") print(f"{'Radius of Gyration (in.):':<30}{ry:.3f}") print(f"{'Eccentricity (in.):':<30}{x_bar:.3f}") print(f"{'Connection Length (in.):':<30}{l:.3f}") print(f"{'Resistance Factor for Tension:':<30}{Phi_T:.3f}") print(f"{'Safety Factor for Tension:':<30}{Omega_T:.3f}") print ("------------------------------------") print ("\n---Required Tensile Strength---") print("Required Factored Strength (Pu) = 1.2 * Pd + 1.6 * Pl") print(f"{'Required Factored Strength (kips):':<30}{Pu:.3f}") print("\nRequired Allowable Strength (Pa) = Pd + Pl") print(f"{'Required Allowable Strength (kips):':<30}{Pa:.3f}") | Prints out all of the variables and the corresponding value that it pairs with. |
| print ("\n---Tensile Yielding---") print ("\n---CHECK: Available Strength > Required Strength---") if LRFD_1 >= Pu: print(f"Load and Resistance Factor Design: {LRFD_1:.3f} kips >= {Pu} kips --> PASS") else: print(f"Load and Resistance Factor Design: {LRFD_1:.3f} kips < {Pu} kips --> FAILS") # ASD Check if ASD_1 >= Pa: print(f"Allowable Strength Design: {ASD_1:.3f} kips >= {Pa} kips --> PASS") else: print(f"Allowable Strength Design: {ASD_1:.3f} kips < {Pa} kips --> FAILS") | An if else function is used to determine if the yield passes or fails. If LRFD is greater or equal to Pu, the yield passes, and if ASD is greater than or equal to Pa, then it passes also. |
| # Determine which U is bigger/Which to use print ("\n---Calculate Shear Lag Factor (U)---\n") U_Calc1 = (2 * bf * tf) / Ag U_Calc2 = 1 - (x_bar / l) print("U (case 2) = 1 - (x_bar / l)") print (f"Value for Case 2: {U_Calc2:.3f}") print("U (case 7) = 0.85 if Flange Width < (2/3) * Depth\n if not U = 0.90") if bf < (2/3) * d: U_Calc3 = 0.85 else: U_Calc3 = 0.90 print (f"Value for Case 7: {U_Calc3}\n") U = max(U_Calc2, U_Calc3) print (f"We will use the larger U = {U:.3f}") | Calculates a ratio of flange are to gross area, and then a AISC Case 2 calculation. An if, else statement is used to determine if the shear lag factor is 0.85 or 0.90 by the width to depth ratio. Then it selects the max U from the AISC case 2 and the if, else statement as the shear lag factor, and prints the value. |
| print("\n---Calculate Net Area (An)---") An = Ag - 4*(dh + 1/16) *tf print("An = Ag - 4*(dh + 1/16) *tf") print (f"An = {An:.3f}\n") print("---Calculate Effective Net Area (Ae)---") Ae = An * U print("Ae = An * U") print (f"An = {An:.3f}\n") | Net Area is found by subtracting 4 bolt holes from the total area. Which the formula is shown, and the final value is displayed. The Effective Net Area is found by multiplying the net area by the shear lag factor and is shown with the formula and result. |
| print("---Check Tensile Rupture Strength (e/Ag)---") TRS = Ae / Ag print("TRS = Ae / Ag") if TRS >= 0.75: print(f"Tensile Rupture Strength: {TRS:.3f} >= 0.75 --> PASS") else: print(f"Tensile Rupture Strength: {TRS:.3f} < 0.75 --> FAIL --> Use AISC Specification Section D2") print ("\nOnly Use AISC Specification Section D2 if TRS Failed\n") | The efficiency value is solved for and the formula is displayed. An if, else statement is presented where if TRS is greater than or equal to 0.75, then it passes, but if not, it fails. |
| print ("---Calulating Pn---") Pn = Fu*Ae print("Pn = Fu*Ae") print (f"Pn = {Pn:.3f} Kips\n") | Nominal Strength is calculated by multiplying the Ultimate Tensile Stress and the Effective Net Area. This is then displayed with the formula and the result. |
| print ("AISC Specification Sec. D2: Available Tensile Rupture Strength is:\n") phi_T = 0.75 Omega_T = 2.00 LRFD2 = phi_T * Pn ASD2 = Pn / Omega_T | The reduction factor is 0.75 and then assigns it. The ASD safety factor for rupture is 2.00 and assigns it. The LRFD is calculated by multiplying the nominal strength by 0.75, and the ASD allowable strength is calculated by dividing the nominal strength by 2.00. |
| if LRFD2 >= Pu: print(f"Load and Resistance Factor Design: {LRFD2:.3f} kips >= {Pu} kips --> PASS") else: print(f"Load and Resistance Factor Design: {LRFD2:.3f} kips < {Pu} kips --> FAILS") # ASD Check if ASD2 >= Pa: print(f"Allowable Strength Design: {ASD2:.3f} kips >= {Pa} kips --> PASS") else: print(f"Allowable Strength Design: {ASD2:.3f} kips < {Pa} kips --> FAILS") | An if-else statement is used to determine if it passes or fails for the LRFD and ASD2. If LRFD2 is greater than or equal to Pu, then it passes, and if ASD2 is greater than or equal to Pa, then it also passes. |
| print ("\n---Checking Recommended SLenderness Limit---\n") Slend = (25 * 12) / ry print("Slenderness = (25 * 12) / ry") if Slend < 300: print(f" {Slend:.3f} < 300 --> PASS") else: print(f" {Slend:.3f} > 300 --> FAILS") | Calculates the slenderness by multiplying 25 * 12 and dividing by ry, and the formula is displayed. An if-else statement is used to see if the slenderness test passes. If it is less than 300, it passes. |
| # --- NEW: Save the final Yield Strength data for plotting --- # We use a short name for the X-axis to keep the chart clean scenario_short_name = s["Name"].split(":")[0] # Store LRFD results plot_data.append({"Scenario": scenario_short_name, "Design Method": "LRFD", "Type": "Demand (Pu)", "Force (kips)": Pu}) plot_data.append({"Scenario": scenario_short_name, "Design Method": "LRFD", "Type": "Capacity (\u03C6Pn)", "Force (kips)": LRFD_1}) # Store ASD results plot_data.append({"Scenario": scenario_short_name, "Design Method": "ASD", "Type": "Demand (Pa)", "Force (kips)": Pa}) plot_data.append({"Scenario": scenario_short_name, "Design Method": "ASD", "Type": "Capacity (Pn/\u03A9)", "Force (kips)": ASD_1}) # ========================================================= # --- NEW: GENERATE THE PLOTLY BAR CHART --- # ========================================================= # Convert our list of data into a Pandas DataFrame df_plot = pd.DataFrame(plot_data) # Create a grouped bar chart, separated into LRFD and ASD columns fig = px.bar( df_plot, x="Scenario", y="Force (kips)", color="Type", facet_col="Design Method", # This splits the chart into LRFD on the left, ASD on the right barmode="group", # Puts Demand and Capacity bars side-by-side text_auto='.1f', # Automatically puts the numbers on top of the bars title="Structural Check: Demand vs. Capacity (Yielding)", color_discrete_sequence=['#EF553B', '#00CC96'] # Demand = Red/Orange, Capacity = Green ) # Clean up the look of the graph fig.update_traces(textposition='outside') fig.update_layout(yaxis_title="Force (kips)", yaxis_range=[0, 500]) fig.for_each_annotation(lambda a: a.update(text=a.text.split("=")[-1])) # Cleans up the LRFD/ASD titles at the top # Show the interactive graph! fig.show() | The results are logged by using plot_data.append. The df plot lines converts the logged results in a table, and then it is turned into a bar chart. The charts are split into LRFD and ASD charts. Barmode is able to put the bars in a side-by-side comparison. And are color coded into orange for demand, and green for capacity. And then fig.show() shows the charts |